RECOMMENDED PERIODIC MAINTENANCE SCHEDULE

Weekly/Monthly (Based on frequency of riding/riding conditions)

Clean bicycle thoroughly.

Check trueness of wheels – radial and lateral.

Check for play in all bearings.

Inspect frame and rims for cracks.

Clean and lube drivetrain.

Rim brakes: clean and rough up brake pads.

Disc brakes: check disc caliper alignment.

Lubricate chain and derailleur pivots.

Check air pressure in shocks.

Monthly (Or every 2000 miles/3219 km)

True wheels - radial, lateral, and dish.

Adjust bearings, as needed.

Adjust brake pads (replace if necessary).

Check cable housings and grease cables.

Check chain for wear.

Check frame for cracks.

Check air pressure in shocks.

Lubricate clipless pedals.

Tighten suspension pivots.

Every Six Months, Annually, or As Needed

Overhaul and replace all bearings.

Replace chain.

Replace cables and housings and grease cables.

Change oil in fork, bushings and seals.

Disassemble and clean drivetrain.

True wheels – radial, lateral, dish, and tension.

Re-grease all nut and bolt fittings (replace if rusted).

Change oil in fork and check for bushing wear.

Cantilever or linear pull brakes: remove brakes and lubricate pivots.

Disc brakes: change brake fluid (hydraulic), check pads (both hydraulic and cable).

RECOMMENDED PARTS LIST

Without an In-House Mechanic

Stock

Tires

Tubes

Pedals

Toe clips and straps

Replacement pins for flat pedals

Brake pads

Grips

Assorted fasteners for racks, bottle cages, and fenders

Saddles

Purchase as Needed

Handlebars

Seatposts

Wheels (front and rear)

With an In-House Mechanic

(In addition to the above; only qualified mechanics should install and/or repair these items due to the risk of rider injury if the repairs are done incorrectly.)

Stock

Spokes (of correct length and gauge)

Bearings

Cables and housing (brake and shifter)

Ferrules for cable housing

Cable end-caps

Chains*

Front chainrings*

Rear cassettes*

*These drivetrain items should be replaced at the same time.

Purchase as Needed

Rear derailleurs

Forks

Crank arms

Bottom brackets